YUKEN INDIA Ltd.

Summer Internship — PCB Design Project Report

Industrial Condition Monitoring Board

A purely Analog PCB Design

| Intern | Avyukth M A |
| --- | --- |
| Programme | B.Tech ECE, VIT Chennai |
| Host Organisation | Yuken India Ltd., CSD Department |
| Design Tool | KiCad 10 |
| Dates | May 25 – June 06, 2026 |

# 1. Abstract

This report documents the design and development of the Industrial Condition Monitoring Board, a printed circuit board (PCB) created as the culminating project of the Analog Electronics phase of the internship at Yuken India Ltd.

The board monitors three independent physical parameters — temperature, pressure and ambient light — using purely analog circuitry. All decision-making is performed without a microcontroller, demonstrating the application of operational amplifiers, comparators, and transistor-based logic to real-world industrial monitoring scenarios.

The design was completed in KiCad 10, progressing through schematic capture, PCB layout, design rule verification (DRC/ERC), and Gerber file generation. The board achieved 0 errors and 0 warnings on final verification.

The complete project, including design documentation, KiCad schematic files, PCB layouts, manufacturing outputs, and project reports, is available at the following GitHub repository: GitHub Repository Link.

# 2. Project Objectives

The project was designed to consolidate and demonstrate the following competencies developed during the Analog Electronics internship weeks:

- Apply operational amplifier theory — non-inverting amplifier gain configuration

- Implement comparator-based threshold detection for multi-level classification

- Design Schmitt trigger circuits with controlled hysteresis

- Perform signal conditioning — low-pass filtering, amplification, and reference generation

- Construct analog decision logic using discrete transistors (no digital ICs)

- Execute a complete PCB design workflow: schematic → layout → DRC → Gerber output

- Work with mixed-technology PCB — SMD passives/ICs and through-hole connectors/transistors

- Produce a manufacturable PCB with organised layout, clean routing, and a ground plane

# 3. System Overview

The board contains three independent monitoring channels, each processing a sensor input through signal conditioning and comparator logic to drive a set of colour-coded LED indicators. A common power supply section generates a regulated 5 V rail from a 12 V DC input.

| Channel | Sensor | Indicators | Key Technique |
| --- | --- | --- | --- |
| Temperature | LM35 (external) | Green / Yellow / Red LEDs | Op-Amp amplification + dual comparators + transistor AND logic |
| Pressure | Potentiometer / analog signal (external) | Green / Yellow / Red LEDs | Resistor ladder references + dual comparators + transistor AND logic |
| Light | LDR (external) | Green / Red LEDs | Voltage divider + Schmitt trigger hysteresis |

The design intentionally excludes microcontrollers, FPGAs, and digital logic ICs. All threshold decisions, zone detection, and output control are performed by analog components.

# 4. Power Supply Section

## 4.1  Input and Protection

The board accepts a 12 V DC input through a 2-pin terminal block (J1). A 1N5819 Schottky diode (D1) provides reverse polarity protection. The 1N5819 was selected for its low forward voltage drop (~0.3 V), minimizing power dissipation while protecting the downstream circuitry.

## 4.2  Voltage Regulation

An LM7805 linear voltage regulator produces a regulated 5 V output. Decoupling capacitors on both the input and output sides of the regulator ensure stability and ripple suppression:

| Location | Capacitor | Purpose |
| --- | --- | --- |
| Input side | 100 nF ceramic | High-frequency noise bypass |
| Input side | 10 µF electrolytic | Bulk decoupling, ripple reduction |
| Output side | 100 nF ceramic | High-frequency noise bypass |
| Output side | 10 µF electrolytic | Load transient response |

A power indicator LED (D2) with a 680 Ω current-limiting resistor (R1) confirms 5 V rail presence. The LED operating current is approximately 4.4 mA.

# 5. Temperature Monitoring Channel

## 5.1  Sensor Interface

An external LM35 precision temperature sensor connects via a 3-pin terminal block (J2: +5 V, signal, GND). The LM35 produces a linear output of 10 mV/°C, requiring amplification to span a useful comparison range.

## 5.2  Amplifier Stage — U2A (LM358)

The first op-amp section (U2A) is configured as a non-inverting amplifier:

| Parameter | Value | Notes |
| --- | --- | --- |
| Configuration | Non-inverting amplifier | — |
| Feedback resistor Rf | 40 kΩ | R2 |
| Input resistor Rin | 10 kΩ | R3 |
| Gain Av | 5 V/V | Av = 1 + Rf/Rin = 1 + 40k/10k |
| Output at 40°C | 2 V | 40°C × 10 mV/°C × 5 = 2 V |
| Output at 60°C | 3 V | 60°C × 10 mV/°C × 5 = 3 V |

## 5.3  Low-Pass Filter

A first-order RC low-pass filter (R = 10 kΩ, C = 1 µF) with a cutoff frequency of approximately 16 Hz is placed before the comparator inputs to suppress high-frequency noise from the sensor signal and power supply.

## 5.4  Threshold Detection — U3 (LM339)

Two comparator sections of the LM339 quad comparator IC (U3) implement dual-threshold detection:

| Comparator | Reference Voltage | Temperature Equivalent | Output Net | Active When |
| --- | --- | --- | --- | --- |
| U3A | 2 V | 40°C | TEMP_GT_40_N | Temperature > 40°C |
| U3B | 3 V | 60°C | TEMP_GT_60_N | Temperature > 60°C |

## 5.5  LED Output Logic — Q1, Q2 (BC547)

Three zones are indicated with colour-coded LEDs. The green and red LEDs respond directly to the comparator outputs. The yellow LED (warning zone: 40–60°C) requires a logical AND of the two comparator outputs, implemented with two BC547 NPN transistors:

- Q1 (driver): switched ON by TEMP_GT_40_N — enables current path for yellow LED

- Q2 (inhibit): switched ON by TEMP_GT_60_N — pulls Q1 base LOW, disabling yellow LED

| Condition | Green LED | Yellow LED | Red LED |
| --- | --- | --- | --- |
| T < 40°C | ON | OFF | OFF |
| 40°C ≤ T < 60°C | OFF | ON | OFF |
| T ≥ 60°C | OFF | OFF | ON |

# 6. Pressure Monitoring Channel

## 6.1  Sensor Interface

An external analog signal (0–5 V range) connects via J4, representing industrial pressure. A potentiometer is used for simulation and testing. No amplification is required since the input already spans the full 5 V reference range.

## 6.2  Signal Conditioning

A first-order RC low-pass filter (R = 10 kΩ, C = 1 µF) with a cutoff frequency of approximately 16 Hz smooths the input before it reaches the comparator network, preventing transient noise from causing false threshold crossings.

## 6.3  Reference Voltage Ladder

Three equal 10 kΩ resistors (R12, R13, R14) form a voltage divider across the 5 V rail, generating two reference points:

| Reference | Voltage | Represents | Used By |
| --- | --- | --- | --- |
| Lower threshold | 1.67 V (5/3) | Lower third of pressure range | Comparator U3C |
| Upper threshold | 3.33 V (10/3) | Upper third of pressure range | Comparator U3D |

## 6.4  LED Output Logic — Q3, Q4 (BC547)

Identical transistor logic to the temperature channel is used for yellow LED zone detection (1.67 V – 3.33 V), with Q3 as the driver and Q4 as the inhibit transistor.

| Condition | Green LED | Yellow LED | Red LED |
| --- | --- | --- | --- |
| Signal < 1.67 V | ON | OFF | OFF |
| 1.67 V ≤ Signal < 3.33 V | OFF | ON | OFF |
| Signal ≥ 3.33 V | OFF | OFF | ON |

# 7. Light Monitoring Channel

## 7.1  Sensor Interface and Voltage Divider

An external LDR (Light Dependent Resistor) connects via J3. On the PCB, a 10 kΩ fixed resistor forms a voltage divider with the LDR between 5 V and GND. The mid-point voltage (LIGHT_SIGNAL) varies inversely with illumination — bright conditions produce a higher voltage, dark conditions a lower voltage.

## 7.2  Schmitt Trigger — U2B (LM358)

The second op-amp section (U2B) is configured as an inverting Schmitt trigger to provide hysteresis, preventing LED flicker at the switching threshold:

| Parameter | Value | Notes |
| --- | --- | --- |
| Reference voltage | 2.5 V | 10k/10k divider from 5 V rail |
| Positive feedback resistor | 100 kΩ | R20 — sets hysteresis band |
| Output net | LIGHT_CONDITION | HIGH = bright, LOW = dark |

The positive feedback through the 100 kΩ resistor creates a hysteresis band around the 2.5 V reference, ensuring the output switches cleanly between states without oscillation near the threshold.

## 7.3  LED Output

- Green LED: illuminated when LIGHT_CONDITION is HIGH (bright environment)

- Red LED: illuminated when LIGHT_CONDITION is LOW (dark environment)

# 8. Bill of Materials — Key Components

| Reference | Component | Value / Type | Package | Quantity | Function |
| --- | --- | --- | --- | --- | --- |
| U1 | LM7805 | 5 V regulator | TO-220 | 1 | Power regulation |
| U2 | LM358 | Dual op-amp | SOIC-8 | 1 | Amplifier + Schmitt trigger |
| U3 | LM339 | Quad comparator | SOIC-14 | 1 | All threshold detection |
| D1 | 1N5819 | Schottky diode | DO-41 | 1 | Reverse polarity protection |
| Q1–Q4 | BC547 | NPN transistor | TO-92 | 4 | Yellow LED AND logic |
| D2–D10 | LED | Various colours | 1206 SMD | 9 | Status indicators |
| R (various) | Resistor | 10k / 20k / 40k / 100k / 680Ω | 0805 SMD | 31 | Biasing, filtering, limiting |
| C1–C7 | Capacitor | 100nF / 1µF / 10µF | 0805 / 1206 | 7 | Decoupling, filtering |
| J1–J4 | Terminal block | Phoenix 5.08mm pitch | Through-hole | 4 | External connections |

# 9. Schematic Design

# 10. PCB Design Details

## 10.1  Layer Stack-up

| Layer | Usage |
| --- | --- |
| Front Copper (F.Cu) | All signal routing, 5 V and 12 V power traces |
| Back Copper (B.Cu) | Solid ground plane — uninterrupted copper pour |
| Front Silkscreen | Component references, polarity markers, connector labels |
| Front/Back Solder Mask | Solder mask to prevent bridging |

## 10.2  Trace Widths

| Net Type | Trace Width | Rationale |
| --- | --- | --- |
| Signal traces | 0.25 mm | Low current signal routing |
| 5 V distribution | 0.5 mm | Multiple load currents |
| 12 V input / regulator | 0.8 – 1.0 mm | Higher current carrying requirement |

## 10.3  Component Placement

The board is organised into functional blocks for clarity and signal flow optimisation:

| Board Region | Block |
| --- | --- |
| Top-left | Power supply — J1, D1, U1, decoupling capacitors, power LED |
| Upper-centre | Temperature channel — J2, U2A, U3A/B, Q1, Q2, LEDs |
| Upper-right | Light channel — J3 (bottom), U2B, light LEDs |
| Lower-centre | Pressure channel — J4, U3C/D, Q3, Q4, LEDs |
| Right edge | All LED indicators grouped for easy observation |

## 10.4  Grounding Strategy

A solid copper ground plane on the bottom layer provides a low-impedance return path for all circuits, reduces noise coupling between channels, and simplifies routing by eliminating the need to route individual GND traces.

## 10.5  Design Verification

| Check | Result |
| --- | --- |
| Electrical Rules Check (ERC) | 0 Errors, 0 Warnings |
| Design Rules Check (DRC) | 0 Errors, 0 Warnings |
| Gerber file generation | Complete — all 9 layers exported |
| Drill file | Generated (.drl format) |

# 11. PCB Renders

## 11.1  3D Top View

Figure 1 — Industrial Condition Monitoring Board (KiCad 3D Render, Top View)

## 11.2  PCB Layout (Top + Bottom Overlay)

Figure 2 — PCB Layout showing front copper (red), back ground plane (blue), and silkscreen

# 12. Internship Learning Outcomes

This project served as a practical integration of the Analog Electronics topics covered during Weeks 1–2 of the internship. The following competencies were applied directly in the design:

| Skill Area | Application in This Project |
| --- | --- |
| Op-amp gain design | Non-inverting amplifier (gain = 5) for LM35 signal conditioning |
| Comparator circuit design | Dual-threshold detection in temperature and pressure channels |
| Schmitt trigger / hysteresis | Light channel — noise-immune on/off switching with 100 kΩ feedback |
| Signal filtering | RC low-pass filters (10 kΩ / 1 µF) on all sensor inputs |
| Voltage reference generation | Resistor dividers for 2 V, 3 V, 1.67 V, 3.33 V, and 2.5 V references |
| Transistor switching logic | BC547 AND logic for zone-specific LED control |
| PCB design workflow | KiCad schematic → layout → DRC → Gerber — complete flow executed |
| Mixed-technology PCB | SMD components alongside through-hole connectors and transistors |
| Ground plane design | Solid B.Cu copper pour for noise reduction and clean return paths |

# 13. Conclusion

The Industrial Condition Monitoring Board demonstrates that industrial-grade multi-parameter monitoring can be achieved entirely with analog circuitry — without a microcontroller. The design integrates operational amplifiers, comparators, transistor logic, and signal conditioning techniques into a single, cohesive PCB.

The project successfully passed all ERC and DRC checks in KiCad 10, and a complete Gerber file set has been generated, making the board ready for prototype manufacturing. The design serves as a practical demonstration of the Analog Electronics concepts covered during the Yuken India Ltd. internship and reflects a solid understanding of real-world circuit design practice.

Possible future enhancements to this design include:

- Adding a 4–20 mA current loop input stage for industrial sensor compatibility

- Integrating a buzzer driver for audible alerts at critical thresholds

- Designing a Rev B with adjustable threshold potentiometers on-board

- Extending the light channel to a multi-level LDR comparator arrangement

# 14. References

[1] Texas Instruments, "LM35 Precision Centigrade Temperature Sensors,"

Datasheet SNIS159H, Texas Instruments, Revised Aug. 2017.

[Online]. Available: https://www.ti.com/lit/ds/symlink/lm35.pdf

[2] Texas Instruments, "LM358, LM358A, LM2904, LM2904A Dual Operational

Amplifiers," Datasheet SNOSBT3, Texas Instruments.

[Online]. Available: https://www.ti.com/lit/ds/symlink/lm358.pdf

[3] Texas Instruments, "LM339, LM339A, LM2901 Family of Differential

Comparators," Datasheet SNOSC16, Texas Instruments.

[Online]. Available: https://www.ti.com/lit/ds/symlink/lm339.pdf

[4] onsemi, "MC7805CT, MC7805BT 1.0 A Positive Voltage Regulators,"

Datasheet, onsemi.

[Online]. Available: //www.onsemi.com/pdf/datasheet/mc7800-d.pdf

[5] onsemi, "BC546, BC547, BC548, BC549, BC550 General Purpose
     Transistors,"Datasheet, onsemi.

[Online]. Available: https://www.onsemi.com/pdf/datasheet/bc546-d.pdf

[6] Diodes Incorporated, "1N5819 — 1.0 A Schottky Barrier Rectifier,"

Datasheet DS23001, Diodes Incorporated.

[Online]. Available: https://www.diodes.com/assets/Datasheets/ds23001.pdf

[7] KiCad EDA, "KiCad 10 Documentation," KiCad Project, 2024.

[Online]. Available: https://docs.kicad.org