YUKEN INDIA Ltd.

Summer Internship — PCB Design Project Report

Industrial Condition Monitoring Board

A purely Analog PCB Design

| Candidate Name | Avyukth M A |
| --- | --- |
| Degree Program | B.Tech, Electronics and Communication Engineering (ECE) |
| Institution | Vellore Institute of Technology, Chennai (VIT Chennai) |
| Batch | 2024 – 2028 |
| Hosting Organization | YUKEN INDIA LTD |
| Supervising Mentor | Mr. Somu Sundar |
| Internship Period | 25-05-2026 to 20-06-2026 |
| Design Tool | KiCAD 10 |
| Project Repository | github.com/Avyukth-ma/Electronics-Projects |
| Report Date | June 25, 2026 |

# 1. Abstract

This report documents the design and development of the Industrial Condition Monitoring Board, a printed circuit board (PCB) created as the culminating project of the Analog Electronics phase of the internship at Yuken India Ltd.

The board monitors three independent physical parameters — temperature, pressure and ambient light — using purely analog circuitry. All decision making is performed without a microcontroller, demonstrating the application of operational amplifiers, comparators, and transistor based logic to real-world industrial monitoring scenarios.

The design was completed in KiCad 10, progressing through schematic capture, PCB layout, design rule verification (DRC/ERC), and Gerber file generation. The board achieved 0 errors and 0 warnings on final verification.

The complete project, including design documentation, KiCad schematic files, PCB layouts, manufacturing outputs, and project reports, is available at the following GitHub repository folder: GitHub Repository Folder Link.

# 2. Project Objectives

The project was designed to consolidate and demonstrate the following competencies developed during the Analog Electronics internship weeks:

- Apply operational amplifier theory, non-inverting amplifier gain configuration

- Implement comparator-based threshold detection for multi-level classification

- Design Schmitt trigger circuits with controlled hysteresis

- Perform signal conditioning such as low-pass filtering, amplification, and reference generation

- Construct analog decision logic using discrete transistors (no digital ICs)

- Execute a complete PCB design workflow: schematic → layout → DRC → Gerber output

- Work with mixed-technology PCB — SMD passives/ICs and through-hole connectors/transistors

- Produce a manufacturable PCB with organised layout, clean routing, and a ground plane

# 3. System Overview

The board contains three independent monitoring channels, each processing a sensor input through signal conditioning and comparator logic to drive a set of colour-coded LED indicators. A common power supply section generates a regulated 5 V rail from a 12 V DC input.

| Channel | Sensor | Indicators | Key Technique |
| --- | --- | --- | --- |
| Temperature | LM35 (external) | Green / Yellow / Red LEDs | Op-Amp amplification + dual comparators + transistor AND logic |
| Pressure | Potentiometer / analog signal (external) | Green / Yellow / Red LEDs | Resistor ladder references + dual comparators + transistor AND logic |
| Light | LDR (external) | Green / Red LEDs | Voltage divider + Schmitt trigger hysteresis |

The design intentionally excludes microcontrollers, FPGAs, and digital logic ICs. All threshold decisions, zone detection, and output control are performed by analog components.

# 4. Power Supply Section

## 4.1  Input and Protection

The board accepts a 12 V DC input through a 2-pin terminal block (J1). A 1N5819 Schottky diode (D1) provides reverse polarity protection. The 1N5819 was selected for its low forward voltage drop (~0.3 V), minimizing power dissipation while protecting the downstream circuitry.

## 4.2  Voltage Regulation

An LM7805 linear voltage regulator produces a regulated 5 V output. Decoupling capacitors on both the input and output sides of the regulator ensure stability and ripple suppression:

| Location | Capacitor | Purpose |
| --- | --- | --- |
| Input side | 100 nF ceramic | High-frequency noise bypass |
| Input side | 10 µF electrolytic | Bulk decoupling, ripple reduction |
| Output side | 100 nF ceramic | High-frequency noise bypass |
| Output side | 10 µF electrolytic | Load transient response |

A power indicator LED (D2) with a 680 Ω current-limiting resistor (R1) confirms 5 V rail presence. The LED operating current is approximately 4.4 mA.

# 5. Temperature Monitoring Channel

## 5.1  Sensor Interface

An external LM35 precision temperature sensor connects via a 3-pin terminal block (J2: +5 V, signal, GND). The LM35 produces a linear output of 10 mV/°C, requiring amplification to span a useful comparison range.

## 5.2  Amplifier Stage — U2A (LM358)

The first op-amp section (U2A) is configured as a non-inverting amplifier:

| Parameter | Value | Notes |
| --- | --- | --- |
| Configuration | Non-inverting amplifier | — |
| Feedback resistor Rf | 40 kΩ | R2 |
| Input resistor Rin | 10 kΩ | R3 |
| Gain Av | 5 V/V | Av = 1 + Rf/Rin = 1 + 40k/10k |
| Output at 40°C | 2 V | 40°C × 10 mV/°C × 5 = 2 V |
| Output at 60°C | 3 V | 60°C × 10 mV/°C × 5 = 3 V |

## 5.3  Low-Pass Filter

A first-order RC low-pass filter (R = 10 kΩ, C = 1 µF) with a cutoff frequency of approximately 16 Hz is placed before the comparator inputs to suppress high-frequency noise from the sensor signal and power supply.

## 5.4  Threshold Detection — U3 (LM339)

Two comparator sections of the LM339 quad comparator IC (U3) implement dual-threshold detection:

| Comparator | Reference Voltage | Temperature Equivalent | Output Net | Active When |
| --- | --- | --- | --- | --- |
| U3A | 2 V | 40°C | TEMP_GT_40_N | Temperature > 40°C |
| U3B | 3 V | 60°C | TEMP_GT_60_N | Temperature > 60°C |

## 5.5  LED Output Logic — Q1, Q2 (BC547)

Three zones are indicated with colour-coded LEDs. The green and red LEDs respond directly to the comparator outputs. The yellow LED (warning zone: 40–60°C) requires a logical AND of the two comparator outputs, implemented with two BC547 NPN transistors:

- Q1 (driver): switched ON by TEMP_GT_40_N, enables current path for yellow LED

- Q2 (inhibit): switched ON by TEMP_GT_60_N, pulls Q1 base LOW, disabling yellow LED

| Condition | Green LED | Yellow LED | Red LED |
| --- | --- | --- | --- |
| T < 40°C | ON | OFF | OFF |
| 40°C ≤ T < 60°C | OFF | ON | OFF |
| T ≥ 60°C | OFF | OFF | ON |

# 6. Pressure Monitoring Channel

## 6.1  Sensor Interface

An external analog signal (0–5 V range) connects via J4, representing industrial pressure. A potentiometer is used for simulation and testing. No amplification is required since the input already spans the full 5 V reference range.

## 6.2  Signal Conditioning

A first-order RC low-pass filter (R = 10 kΩ, C = 1 µF) with a cutoff frequency of approximately 16 Hz smooths the input before it reaches the comparator network, preventing transient noise from causing false threshold crossings.

## 6.3  Reference Voltage Ladder

Three equal 10 kΩ resistors (R12, R13, R14) form a voltage divider across the 5 V rail, generating two reference points:

| Reference | Voltage | Represents | Used By |
| --- | --- | --- | --- |
| Lower threshold | 1.67 V (5/3) | Lower third of pressure range | Comparator U3C |
| Upper threshold | 3.33 V (10/3) | Upper third of pressure range | Comparator U3D |

## 6.4  LED Output Logic — Q3, Q4 (BC547)

Identical transistor logic to the temperature channel is used for yellow LED zone detection (1.67 V – 3.33 V), with Q3 as the driver and Q4 as the inhibit transistor.

| Condition | Green LED | Yellow LED | Red LED |
| --- | --- | --- | --- |
| Signal < 1.67 V | ON | OFF | OFF |
| 1.67 V ≤ Signal < 3.33 V | OFF | ON | OFF |
| Signal ≥ 3.33 V | OFF | OFF | ON |

# 7. Light Monitoring Channel

## 7.1  Sensor Interface and Voltage Divider

An external LDR (Light Dependent Resistor) connects via J3. On the PCB, a 10 kΩ fixed resistor forms a voltage divider with the LDR between 5 V and GND. The mid-point voltage (LIGHT_SIGNAL) varies inversely with illumination i.e bright conditions produce a higher voltage, dark conditions a lower voltage.

## 7.2  Schmitt Trigger — U2B (LM358)

The second op-amp section (U2B) is configured as an inverting Schmitt trigger to provide hysteresis, preventing LED flicker at the switching threshold:

| Parameter | Value | Notes |
| --- | --- | --- |
| Reference voltage | 2.5 V | 10k/10k divider from 5 V rail |
| Positive feedback resistor | 100 kΩ | R20 sets hysteresis band |
| Output net | LIGHT_CONDITION | HIGH = bright, LOW = dark |

The positive feedback through the 100 kΩ resistor creates a hysteresis band around the 2.5 V reference, ensuring the output switches cleanly between states without oscillation near the threshold.

## 7.3  LED Output

- Green LED: illuminated when LIGHT_CONDITION is HIGH (bright environment)

- Red LED: illuminated when LIGHT_CONDITION is LOW (dark environment)

# 8. Bill of Materials — Key Components

| Reference | Component | Value / Type | Package | Quantity | Function |
| --- | --- | --- | --- | --- | --- |
| U1 | LM7805 | 5 V regulator | TO-220 | 1 | Power regulation |
| U2 | LM358 | Dual op-amp | SOIC-8 | 1 | Amplifier + Schmitt trigger |
| U3 | LM339 | Quad comparator | SOIC-14 | 1 | All threshold detection |
| D1 | 1N5819 | Schottky diode | DO-41 | 1 | Reverse polarity protection |
| Q1–Q4 | BC547 | NPN transistor | TO-92 | 4 | Yellow LED AND logic |
| D2–D10 | LED | Various colours | 1206 SMD | 9 | Status indicators |
| R (various) | Resistor | 10k / 20k / 40k / 100k / 680Ω | 0805 SMD | 31 | Biasing, filtering, limiting |
| C1–C7 | Capacitor | 100nF / 1µF / 10µF | 0805 / 1206 | 7 | Decoupling, filtering |
| J1–J4 | Terminal block | Phoenix 5.08mm pitch | Through-hole | 4 | External connections |

# 9. Schematic Design

# 10. PCB Design Details

## 10.1  Layer Stack-up

| Layer | Usage |
| --- | --- |
| Front Copper (F.Cu) | All signal routing, 5 V and 12 V power traces |
| Back Copper (B.Cu) | Solid ground plane provides uninterrupted copper pour |
| Front Silkscreen | Component references, polarity markers, connector labels |
| Front/Back Solder Mask | Solder mask to prevent bridging |

## 10.2  Trace Widths

| Net Type | Trace Width | Rationale |
| --- | --- | --- |
| Signal traces | 0.25 mm | Low current signal routing |
| 5 V distribution | 0.5 mm | Multiple load currents |
| 12 V input / regulator | 0.8 – 1.0 mm | Higher current carrying requirement |

## 10.3  Component Placement

The board is organised into functional blocks for clarity and signal flow optimisation:

| Board Region | Block |
| --- | --- |
| Top-left | Power supply — J1, D1, U1, decoupling capacitors, power LED |
| Upper-centre | Temperature channel — J2, U2A, U3A/B, Q1, Q2, LEDs |
| Upper-right | Light channel — J3 (bottom), U2B, light LEDs |
| Lower-centre | Pressure channel — J4, U3C/D, Q3, Q4, LEDs |
| Right edge | All LED indicators grouped for easy observation |

## 10.4  Grounding Strategy

A solid copper ground plane on the bottom layer provides a low-impedance return path for all circuits, reduces noise coupling between channels, and simplifies routing by eliminating the need to route individual GND traces.

## 10.5  Design Verification

| Check | Result |
| --- | --- |
| Electrical Rules Check (ERC) | 0 Errors, 0 Warnings |
| Design Rules Check (DRC) | 0 Errors, 0 Warnings |
| Gerber file generation | Completed and all 9 layers exported |
| Drill file | Generated (.drl format) |

# 11. PCB Renders

## 11.1  3D Top View

Figure 1 — Industrial Condition Monitoring Board (KiCad 3D Render, Top View)

## 11.2  PCB Layout (Top + Bottom Overlay)

Figure 2 — PCB Layout showing front copper (red), back ground plane (blue), and silkscreen

# 12. Internship Learning Outcomes

This project served as a practical integration of the Analog Electronics topics covered during Weeks 1–2 of the internship. The following competencies were applied directly in the design:

| Skill Area | Application in This Project |
| --- | --- |
| Op-amp gain design | Non-inverting amplifier (gain = 5) for LM35 signal conditioning |
| Comparator circuit design | Dual-threshold detection in temperature and pressure channels |
| Schmitt trigger / hysteresis | Light channel noise-immune on/off switching with 100 kΩ feedback |
| Signal filtering | RC low-pass filters (10 kΩ / 1 µF) on all sensor inputs |
| Voltage reference generation | Resistor dividers for 2 V, 3 V, 1.67 V, 3.33 V, and 2.5 V references |
| Transistor switching logic | BC547 AND logic for zone-specific LED control |
| PCB design workflow | KiCad schematic → layout → DRC → Gerber. Complete flow executed |
| Mixed-technology PCB | SMD components alongside through-hole connectors and transistors |
| Ground plane design | Solid B.Cu copper pour for noise reduction and clean return paths |

# 13. Conclusion

The Industrial Condition Monitoring Board demonstrates that industrial-grade multi-parameter monitoring can be achieved entirely with analog circuitry without a microcontroller. The design integrates operational amplifiers, comparators, transistor logic, and signal conditioning techniques into a single, cohesive PCB.

The project successfully passed all ERC and DRC checks in KiCad 10, and a complete Gerber file set has been generated, making the board ready for prototype manufacturing. The design serves as a practical demonstration of the Analog Electronics concepts covered during the Yuken India Ltd. internship and reflects a solid understanding of real-world circuit design practice.

Possible future enhancements to this design include:

- Adding a 4–20 mA current loop input stage for industrial sensor compatibility

- Integrating a buzzer driver for audible alerts at critical thresholds

- Designing a Rev B with adjustable threshold potentiometers on-board

- Extending the light channel to a multi-level LDR comparator arrangement

# 14. References

[1] Texas Instruments, "LM35 Precision Centigrade Temperature Sensors,"

Datasheet SNIS159H, Texas Instruments, Revised Aug. 2017.

[Online]. Available: https://www.ti.com/lit/ds/symlink/lm35.pdf

[2] Texas Instruments, "LM358, LM358A, LM2904, LM2904A Dual Operational

Amplifiers," Datasheet SNOSBT3, Texas Instruments.

[Online]. Available: https://www.ti.com/lit/ds/symlink/lm358.pdf

[3] Texas Instruments, "LM339, LM339A, LM2901 Family of Differential

Comparators," Datasheet SNOSC16, Texas Instruments.

[Online]. Available: https://www.ti.com/lit/ds/symlink/lm339.pdf

[4] onsemi, "MC7805CT, MC7805BT 1.0 A Positive Voltage Regulators,"

Datasheet, onsemi.

[Online]. Available: //www.onsemi.com/pdf/datasheet/mc7800-d.pdf

[5] onsemi, "BC546, BC547, BC548, BC549, BC550 General Purpose
     Transistors,"Datasheet, onsemi.

[Online]. Available: https://www.onsemi.com/pdf/datasheet/bc546-d.pdf

[6] Diodes Incorporated, "1N5819 — 1.0 A Schottky Barrier Rectifier,"

Datasheet DS23001, Diodes Incorporated.

[Online]. Available: https://www.diodes.com/assets/Datasheets/ds23001.pdf

[7] KiCad EDA, "KiCad 10 Documentation," KiCad Project, 2024.

[Online]. Available: https://docs.kicad.org

# 15. Declaration and Sign-Off

I hereby declare that this project report is a true and accurate account of the work performed during my internship. The design, implementation, and experimental verification described in this report were carried out by me under the supervision of the stated mentor.

| Developer Name: Avyukth M A Degree: B.Tech ECE (2024–2028) Institution: VIT Chennai | Supervising Mentor Name: Mr. Somu Sundar Title: Senior Manager Organization: Yuken India Ltd |
| --- | --- |